Audio Standards & Specifications

For TV, Film & Real Estate Media Production

AI Audio QC & Enhancement Reference Guide

Proof | Brought to you by Aerial Canvas

# 1. LOUDNESS STANDARDS & METERING

## 1.1 Broadcast Loudness Standards

Loudness is measured in LUFS (Loudness Units Full Scale) or LKFS (Loudness Units K-weighted Full Scale). These two terms are interchangeable and represent the same measurement. The following standards govern loudness for different delivery platforms:

Industry Loudness Targets

| Platform / Standard | Target Loudness | Tolerance | True Peak Max |
| --- | --- | --- | --- |
| ATSC A/85 (US Broadcast TV) | -24 LKFS | ±2 LU | -2 dBTP |
| EBU R128 (European Broadcast) | -23 LUFS | ±1 LU | -1 dBTP |
| Netflix | -27 LUFS (±2 LU) dialogue-gated | See spec | -2 dBTP |
| Amazon Prime Video | -24 LUFS | ±2 LU | -2 dBTP |
| YouTube (normalization) | -14 LUFS (normalized down) | N/A | -1 dBTP |
| Instagram / TikTok / Social | -14 LUFS (recommended) | N/A | -1 dBTP |
| Spotify (reference) | -14 LUFS | N/A | -1 dBTP |
| Apple Music (reference) | -16 LUFS | N/A | -1 dBTP |
| Podcast (general) | -16 to -19 LUFS | ±1 LU | -1 dBTP |
| Real Estate Video (recommended) | -14 to -16 LUFS | ±1 LU | -1 dBTP |

Key Loudness Terminology

- Integrated Loudness: The average loudness over the entire program. This is the primary measurement for compliance.

- Momentary Loudness: Loudness measured over a 400ms window. Used for real-time monitoring.

- Short-Term Loudness: Loudness measured over a 3-second window. Useful for identifying loud/quiet sections.

- Loudness Range (LRA): The difference between the loudest and quietest parts. Indicates dynamic range.

- True Peak (dBTP): The actual peak level including inter-sample peaks. Must never exceed the platform maximum.

- LU (Loudness Unit): A relative unit. 1 LU = 1 dB of loudness difference.

## 1.2 Real Estate Video Loudness Recommendations

Real estate videos are primarily delivered via social media (YouTube, Instagram, Facebook, TikTok) and MLS/listing platforms. The target loudness should be optimized for these platforms:

- Target integrated loudness: -14 to -16 LUFS. This is the sweet spot for social media delivery without being normalized down or sounding quiet.

- True peak ceiling: -1.0 dBTP. This prevents clipping on any playback system and allows headroom for codec compression.

- Loudness range (LRA): 4-8 LU for real estate videos. Keeps the mix consistent without extreme dynamics. Music-only videos can be tighter (3-5 LU).

- Short-term loudness should not exceed -10 LUFS at any point (prevents jarring moments).

- Short-term loudness should not drop below -24 LUFS during content (prevents inaudible sections).

# 2. DIALOGUE & VOICEOVER LEVELS

## 2.1 Broadcast & Film Standards for Dialogue

Dialogue is the anchor of any mix. All other elements (music, sound effects, ambience) are mixed relative to the dialogue level. These are the industry standards:

Dialogue Level Targets

| Context | Dialogue Target | Peak Level | Notes |
| --- | --- | --- | --- |
| Film (theatrical) | -27 to -24 LUFS (dialnorm) | -6 dBFS peak | Wide dynamic range, calibrated rooms |
| TV Broadcast (US) | -24 LKFS (dialogue-gated) | -6 dBFS peak | ATSC A/85 compliant |
| Streaming (Netflix) | -27 LUFS (dialogue-gated) | -2 dBTP | Measured with dialogue gate |
| YouTube / Social Media | -14 to -16 LUFS | -1 dBTP | Optimized for phone/laptop playback |
| Podcast | -16 to -19 LUFS | -1 dBTP | Spoken word focused |
| Real Estate VO | -12 to -14 dBFS average | -3 dBFS peak | Clear, upfront, and intelligible |

Dialogue Quality Checklist (AI QC Flags)

Your AI should flag any of the following dialogue/voiceover issues:

- Level too low: Average dialogue below -18 dBFS or integrated loudness below -20 LUFS. The voiceover will be buried or inaudible on mobile devices.

- Level too hot: Peaks exceeding -1 dBFS or sustained levels above -6 dBFS. Risk of distortion and clipping.

- Inconsistent levels: More than 6 dB variation between the quietest and loudest dialogue passages within the same video. Indicates missing compression.

- Sibilance: Harsh, piercing 's', 'sh', 'ch' sounds in the 4-10 kHz range. Indicates need for de-essing.

- Plosives: Low-frequency pops on 'p', 'b', 't', 'd' sounds (below 100 Hz). Indicates missing pop filter or need for high-pass filtering.

- Room reverb/echo: Audible reflections that make dialogue sound distant or hollow. Common in empty real estate properties. Indicates need for noise reduction or re-recording.

- Background noise: HVAC hum (60/120 Hz), traffic, wind, refrigerator buzz, or other environmental noise louder than -50 dBFS. Indicates need for noise reduction.

- Mouth clicks/lip smacks: Distracting oral noises between words. Can be removed with spectral editing or AI tools.

- Phase issues: Hollow or thin-sounding dialogue that may indicate microphone phase cancellation in stereo recordings.

- Clipping/distortion: Any digital clipping (signal exceeding 0 dBFS) or analog distortion. Unfixable in most cases.

## 2.2 Voiceover Processing Chain (Recommended Order)

For real estate video voiceover, the following signal processing chain produces broadcast-quality results:

- 1. High-Pass Filter (HPF): Set at 80-100 Hz with a 12-18 dB/octave slope. Removes rumble, HVAC hum, handling noise, and plosive energy.

- 2. Noise Reduction: Apply broadband noise reduction to remove room tone, HVAC, and ambient noise. Target noise floor of -60 dBFS or lower.

- 3. De-Essing: Reduce sibilance in the 4-10 kHz range. Threshold should only catch harsh sibilants, not affect normal speech.

- 4. Compression: Ratio of 2:1 to 4:1, threshold set to catch the loudest 6-10 dB of the signal. Attack time 5-15ms (fast enough to catch peaks, slow enough to preserve transients). Release time 50-150ms. Goal: reduce dynamic range to 6-10 dB.

- 5. EQ (Equalization): Presence boost of 1-3 dB around 3-5 kHz for clarity and intelligibility. Optional warmth reduction at 200-400 Hz if boxy sounding. Optional air boost at 10-12 kHz for brightness.

- 6. Limiter: Set ceiling at -1 dBTP. This is the final safety net preventing any peaks from exceeding the maximum.

# 3. MUSIC LEVELS & MIX BALANCE

## 3.1 Music Level Standards Relative to Dialogue

Music in video production is always mixed relative to the dialogue. The balance between music and dialogue is one of the most critical aspects of a professional mix:

Music Level Guidelines

| Scenario | Music Level (relative to dialogue) | Absolute Level Range | Notes |
| --- | --- | --- | --- |
| Music under voiceover (bed) | -15 to -20 dB below dialogue | -30 to -36 dBFS avg | Must not compete with speech intelligibility |
| Music only (no dialogue) | Full level | -14 to -16 LUFS integrated | Match overall program loudness target |
| Music intro before VO starts | Full level, then duck | Start at -16 LUFS, duck to -30 dBFS | 2-3 second fade/duck transition |
| Music outro after VO ends | Swell back to full | Rise from -30 to -16 LUFS | 2-3 second swell transition |
| Emotional/dramatic moment | -10 to -12 dB below dialogue | -24 to -28 dBFS avg | Music slightly more present for impact |
| Transition/montage (no VO) | Full level | -14 to -16 LUFS integrated | Music carries the energy |
| Film/TV underscore | -18 to -24 dB below dialogue | -36 to -42 dBFS avg | Very subtle, atmospheric |
| Real estate walkthrough | -15 to -18 dB below VO | -28 to -33 dBFS avg | Clean and unobtrusive |

Music Ducking / Sidechain Standards

When voiceover and music play simultaneously, music must be ducked (reduced in volume) to maintain dialogue clarity:

- Duck amount: -12 to -18 dB reduction when dialogue is present. The music should be clearly audible but never compete with the voice.

- Duck attack time: 50-200ms. Fast enough to get out of the way when dialogue starts, slow enough to sound natural (not pumping).

- Duck release time: 300-800ms. Slow enough to avoid pumping between words, fast enough to fill gaps in dialogue.

- Hold time: 200-500ms. Keeps the music ducked during brief pauses between sentences.

- Frequency-dependent ducking: Advanced technique where only the mid-range (300 Hz - 3 kHz) of the music is ducked, preserving bass and high-end energy while clearing space for speech frequencies.

## 3.2 Music Selection Quality Flags

Your AI should evaluate the following aspects of background music in real estate videos:

- Genre appropriateness: Real estate videos typically use light acoustic, ambient electronic, soft pop, modern classical, or upbeat indie. Avoid heavy bass, aggressive genres, or anything with lyrics that compete with voiceover.

- Lyrics presence: Instrumental tracks are strongly preferred for real estate. If vocals are present, they should be minimal, ambient, or in a non-primary language. Lyrics in the same language as the voiceover directly compete for intelligibility.

- Tempo match: Music tempo should match the pace of the video edit. Property tours typically range from 80-120 BPM. Luxury properties trend slower (70-90 BPM). Upbeat/modern properties trend faster (100-120 BPM).

- Frequency conflict: Music with heavy mid-range (300 Hz - 3 kHz) competes directly with speech frequencies. Flag tracks where the spectral content overlaps significantly with the voiceover.

- Dynamic consistency: Background music should have a relatively flat dynamic range (3-5 LU LRA). Tracks with sudden volume changes, drops, or builds can be jarring in a property tour context.

- Loop quality: If music loops, the loop point should be seamless with no audible pop, click, or rhythmic discontinuity.

# 4. SOUND EFFECTS & AMBIENCE

## 4.1 Sound Effect Levels

Sound effects (SFX) add polish and production value. In real estate media, they are used sparingly but effectively:

SFX Level Guidelines

| SFX Type | Level (relative to dialogue) | Absolute Level Range | Usage Context |
| --- | --- | --- | --- |
| Transition swooshes/whooshes | -10 to -15 dB below dialogue | -22 to -28 dBFS peak | Scene transitions, room changes |
| UI sounds / subtle tones | -15 to -20 dB below dialogue | -28 to -34 dBFS peak | Text overlays, info graphics |
| Cinematic impacts/hits | -6 to -10 dB below dialogue | -18 to -24 dBFS peak | Opening title, dramatic reveals |
| Drone/camera movement | -20 to -25 dB below dialogue | -34 to -40 dBFS peak | Subtle movement texture |
| Nature ambience (birds, water) | -20 to -30 dB below dialogue | -34 to -44 dBFS avg | Outdoor establishing shots |
| Room tone / silence fill | -40 to -50 dBFS | -40 to -50 dBFS avg | Fill gaps, maintain sonic continuity |
| Door open/close | -12 to -18 dB below dialogue | -26 to -32 dBFS peak | Transitioning between rooms |

SFX Quality Standards

- Sample rate: Minimum 44.1 kHz / 16-bit. Preferred: 48 kHz / 24-bit (matches video production standard).

- No embedded noise: SFX files should have a clean noise floor below -60 dBFS.

- Proper fades: All SFX should have fade-in and fade-out (minimum 5-10ms) to prevent clicks or pops at start/end.

- Stereo width: SFX should be mixed in stereo but not excessively wide. Keep critical elements (impacts, swooshes) centered or slightly panned.

- Frequency range: SFX should not have excessive sub-bass content below 40 Hz (causes rumble on playback systems) unless intentionally designed for impact.

## 4.2 Ambient Sound / Room Tone

Room tone and ambience are essential for professional audio even when they seem silent:

- Every location has a unique room tone (the ambient sound of the space when nothing is happening). This should be captured on-site for 30-60 seconds.

- Room tone is used to fill gaps between dialogue edits, preventing jarring cuts to digital silence.

- Target room tone level: -40 to -50 dBFS. Present but not consciously noticeable.

- Outdoor ambience (wind, birds, traffic) should be consistent throughout outdoor segments. Sudden changes indicate edit points that need smoothing.

- AI flag: Digital silence (absolute zero signal) between edits indicates missing room tone. This sounds unnatural and should be filled.

# 5. TECHNICAL AUDIO SPECIFICATIONS

## 5.1 File Format & Delivery Specs

| Specification | Broadcast Standard | Streaming Standard | Real Estate Recommended |
| --- | --- | --- | --- |
| Sample Rate | 48 kHz | 48 kHz | 48 kHz |
| Bit Depth | 24-bit | 24-bit | 24-bit (16-bit acceptable) |
| Channels | 5.1 or 7.1 surround | Stereo or 5.1 | Stereo (2-channel) |
| Codec (delivery) | PCM / AC-3 / E-AC-3 | AAC-LC / E-AC-3 | AAC-LC (256 kbps+) |
| Codec (master) | WAV / AIFF (uncompressed) | WAV / AIFF | WAV / AIFF |
| True Peak Maximum | -2 dBTP | -1 dBTP | -1 dBTP |
| Integrated Loudness | -24 LKFS (±2) | Per platform | -14 to -16 LUFS |
| Noise Floor | Below -60 dBFS | Below -50 dBFS | Below -50 dBFS |
| Phase Correlation | > 0.5 (stereo) | > 0.5 | > 0.5 |

## 5.2 Frequency Spectrum Guidelines

Understanding how the frequency spectrum is allocated helps ensure a clean, professional mix:

Frequency Allocation by Element

| Frequency Range | Description | Primary Elements | Common Issues |
| --- | --- | --- | --- |
| 20-60 Hz | Sub-bass | Rumble, sub impacts | Remove unless intentional (HPF at 60-80 Hz) |
| 60-200 Hz | Bass | Music bass, warmth, body | Muddiness if too much accumulation |
| 200-500 Hz | Low-mids | Voice body, music body | Boxiness, muddiness in untreated rooms |
| 500 Hz - 2 kHz | Mid-range | Voice fundamental, music melody | Most critical for speech intelligibility |
| 2-5 kHz | Upper-mids / Presence | Voice clarity, consonants | Harshness if over-boosted, sibilance |
| 5-10 kHz | Brilliance | Sibilance, cymbal detail | Sibilance, listener fatigue |
| 10-20 kHz | Air / Sparkle | Breath, shimmer, open feel | Missing if mic is dull, harsh if too bright |

## 5.3 Dynamic Range Guidelines

| Content Type | Recommended LRA | Crest Factor | Notes |
| --- | --- | --- | --- |
| Film (theatrical) | 15-20 LU | 18-20 dB | Full dynamics, calibrated environment |
| TV Drama | 8-12 LU | 14-16 dB | Moderate dynamics for home viewing |
| TV Commercial | 3-5 LU | 8-10 dB | Heavily compressed for consistency |
| YouTube/Social Content | 5-8 LU | 10-14 dB | Moderate compression for varied playback |
| Podcast | 3-6 LU | 8-12 dB | Tight dynamics for voice clarity |
| Real Estate Video | 4-8 LU | 10-14 dB | Consistent but not over-compressed |
| Music Only (mastered) | 6-10 LU | 10-14 dB | Genre dependent, avoid loudness war |

# 6. COMMON AUDIO PROBLEMS & AI DETECTION FLAGS

## 6.1 Problems the AI Should Detect and Flag

The following is a comprehensive list of audio issues your AI should be trained to identify, categorized by severity:

Critical (Must Fix)

- Clipping/Digital Distortion: Signal exceeding 0 dBFS causing hard clipping. Sounds like crackling or buzzing on peaks. Detection: any sample at 0 dBFS or three consecutive samples at maximum value.

- No Audio: Complete silence where audio should exist. Detection: signal below -80 dBFS for more than 2 seconds during content.

- Mono Incompatibility: Audio that phase-cancels when summed to mono (common when played on phone speakers). Detection: phase correlation dropping below -0.5.

- Sample Rate Mismatch: Audio at 44.1 kHz mixed into a 48 kHz timeline (or vice versa) causing pitch shift. Detection: metadata mismatch or pitch analysis.

- Sync Issues: Audio and video out of sync by more than 1 frame (>33ms at 30fps). Detection: compare transients in audio to visual events.

- Extreme Loudness: Integrated loudness exceeding -10 LUFS or below -24 LUFS. Detection: loudness measurement outside acceptable range.

Major (Should Fix)

- Excessive Background Noise: Persistent noise (HVAC, traffic, wind) above -40 dBFS during dialogue. Detection: noise floor analysis during speech pauses.

- Plosives: Low-frequency transient bursts on plosive consonants. Detection: energy spike below 100 Hz coinciding with speech onset.

- Sibilance: Harsh, piercing sibilant sounds. Detection: energy spikes in 4-10 kHz band exceeding 10 dB above average speech level.

- Music Too Loud Under VO: Music level within 10 dB of dialogue level. Detection: compare music-band energy to speech-band energy during simultaneous playback.

- Inconsistent VO Levels: More than 6 dB variation in average dialogue level between segments. Detection: segment-by-segment loudness analysis.

- Abrupt Edits: Hard cuts in audio without crossfades causing audible clicks or pops. Detection: discontinuity in waveform at edit points.

- Reverb/Echo: Excessive room reflections making dialogue sound distant. Detection: RT60 estimation or reverb tail analysis showing decay > 500ms.

- Loudness Non-Compliance: Integrated loudness outside target range for intended platform. Detection: LUFS measurement against platform specification.

Minor (Nice to Fix)

- Mouth Clicks: Brief transient noises from saliva between words. Detection: short-duration (<50ms) broadband clicks during speech pauses.

- Breath Sounds: Audible inhales/exhales between phrases. Detection: energy in 1-4 kHz range during non-speech moments with characteristic breath shape.

- Slight Hum: 60 Hz (US) or 50 Hz (EU) electrical hum at low levels (-50 to -60 dBFS). Detection: narrowband energy at exact mains frequency and harmonics.

- Stereo Imbalance: Dialogue not centered in stereo field (more than 3 dB difference between L/R). Detection: L/R level comparison during speech.

- Music Loop Point Audible: Noticeable repeat point in looping background music. Detection: waveform correlation analysis for repeated patterns with discontinuity.

- Missing Room Tone: Digital silence between edits instead of continuous room tone. Detection: signal dropping to absolute zero between dialogue segments.

# 7. MIX STRUCTURE FOR REAL ESTATE VIDEOS

## 7.1 Typical Mix Architecture

A professional real estate video mix typically contains the following audio layers, each with its own level and processing:

| Layer | Content | Target Level | Processing |
| --- | --- | --- | --- |
| Layer 1 - Voiceover | Narration / agent VO | -12 to -14 dBFS avg | HPF, compression, EQ, de-ess, limiter |
| Layer 2 - Music Bed | Background music (instrumental) | -28 to -33 dBFS avg (under VO) | Sidechain duck from VO, EQ cut 2-4 kHz |
| Layer 3 - Music Full | Music at full level (no VO sections) | -14 to -16 LUFS integrated | Light compression, limiter |
| Layer 4 - SFX | Transitions, whooshes, impacts | -22 to -28 dBFS peak | Fade in/out, HPF |
| Layer 5 - Ambience | Room tone, outdoor atmosphere | -40 to -50 dBFS avg | Continuous, fills gaps |
| Layer 6 - On-Camera | Agent speaking on camera, interviews | -12 to -14 dBFS avg | Same as VO chain + noise reduction |
| Master Bus | Sum of all layers | -14 to -16 LUFS integrated | Gentle compression, limiter at -1 dBTP |

## 7.2 Timeline Structure & Audio Cues

A typical real estate video follows this audio structure:

- 0:00-0:03 - Music intro: Full-level music establishing mood and energy before any dialogue begins.

- 0:03-0:05 - Music duck: Music fades down 12-18 dB as voiceover begins. Transition should take 1-2 seconds.

- 0:05 onward - VO + music bed: Voiceover at -12 to -14 dBFS with music bedded underneath at -28 to -33 dBFS.

- Transition points: Between rooms or scenes, music may briefly swell (1-2 seconds) to full level with a transition SFX, then duck back down when VO resumes.

- Drone/aerial segments: If no VO, music plays at full level. Subtle wind ambience may be layered underneath.

- Final 3-5 seconds: Music swells back to full level for the outro. Contact info/branding on screen.

- End: Music fades out over 2-3 seconds. Clean ending, no abrupt cut.

# 8. PLATFORM-SPECIFIC DELIVERY NOTES

## 8.1 YouTube

- YouTube normalizes audio DOWN to -14 LUFS but does NOT normalize up. If your audio is below -14 LUFS, it will play back quieter than other content.

- Target: -14 LUFS integrated, -1 dBTP true peak.

- Stereo is standard. 5.1 is supported but unnecessary for real estate.

- AAC compression in YouTube's processing can introduce artifacts on heavily limited audio. Leave 1 dB of headroom.

## 8.2 Instagram Reels / Stories

- Instagram compresses audio aggressively. Target -14 LUFS with -1 dBTP.

- Many viewers watch without sound initially. When they unmute, audio should be immediately clear and well-balanced.

- Keep dynamic range tight (LRA under 6 LU) since playback is primarily on phone speakers.

- Bass content below 80 Hz is largely inaudible on phone speakers. Don't rely on sub-bass for impact.

## 8.3 TikTok

- Similar to Instagram. Target -14 LUFS, tight dynamics.

- TikTok's own music/sound library is very loud. Your content should match.

- Short-form content means every second matters - audio should engage immediately.

## 8.4 Facebook

- Facebook normalizes to approximately -16 LUFS.

- Autoplay is muted by default. Ensure the video works visually without sound and that audio is clear when unmuted.

- Target -14 to -16 LUFS for optimal playback.

## 8.5 MLS / Listing Platforms

- No standardized audio normalization. Audio plays back at whatever level it's delivered.

- Target -14 to -16 LUFS to be safe across all embedded players.

- Many MLS platforms use basic HTML5 video players with no audio processing.

- Ensure audio is well-balanced at the source since there's no platform-side correction.

# 9. QUICK REFERENCE CARD

## Real Estate Video Audio Targets (At a Glance)

| Parameter | Target Value | Acceptable Range |
| --- | --- | --- |
| Integrated Loudness | -14 to -16 LUFS | -13 to -17 LUFS |
| True Peak Maximum | -1 dBTP | Never exceed 0 dBTP |
| Dialogue Average | -12 to -14 dBFS | -10 to -16 dBFS |
| Dialogue Peak | -3 dBFS | -1 to -6 dBFS |
| Music Under VO | -28 to -33 dBFS | -25 to -36 dBFS |
| Music Full (no VO) | -14 to -16 LUFS | -12 to -18 LUFS |
| SFX Peaks | -22 to -28 dBFS | -18 to -30 dBFS |
| Room Tone / Ambience | -40 to -50 dBFS | -35 to -55 dBFS |
| Noise Floor | Below -50 dBFS | Below -40 dBFS |
| Loudness Range (LRA) | 4-8 LU | 3-10 LU |
| Phase Correlation | > 0.5 | > 0 |
| Sample Rate | 48 kHz | 44.1 or 48 kHz |
| Bit Depth | 24-bit | 16 or 24-bit |
| Delivery Codec | AAC-LC 256 kbps+ | AAC 192 kbps+ |
| Music Duck Amount | -12 to -18 dB | -10 to -20 dB |
| Duck Attack | 50-200ms | 20-500ms |
| Duck Release | 300-800ms | 200-1500ms |

Document prepared for Proof by Aerial Canvas. All values based on current industry standards including ATSC A/85, EBU R128, and platform-specific recommendations as of 2026.